| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Check Border /Title block (Signatory initials, Revision Phrases, date in title block), Stamp (review/approval/construction), North direction block matches with the project drafting specification and is consistent with the deliverables from other discipline. |  |  |  |
|  | Check changes wrt old revision of drawings and client comments are indicated with proper cloud mark, & Rev. mark. |  |  |  |
|  | Check text properties are followed as per project drafting philosophy / CAD manual. |  |  |  |
|  | Check drawing format as per Project / Client Standards / approval by lead engineer. |  |  |  |
|  | Check legends, abbreviations and notes are defined properly. |  |  |  |
|  | Check notes shown on drawing are relevant and appropriately referenced. |  |  |  |
|  | Check items being modified as existing or new are indicated. |  |  |  |
|  | Check holds if any are indicated on drawing, with proper HOLD clouds. |  |  |  |
|  | Check that no abbreviations are used in Drawing Title. Abbreviations if any are included in the legend part of the drawing. |  |  |  |
|  | Ensure Drawing Title, Rev matches with TDR. Soft file name / path as per project specific requirement are mentioned in drawing. Approval category to be ensured. |  |  |  |
|  | Ensure the drawing is sectionalized to indicate field, package related local control panels & unit control panels unit in the field, equipment rooms, auxiliary room, control room, telecom rooms MCC / Switchgear rooms (substation) etc. as applicable. Other sections as applicable to the project shall be included. |  |  |  |
|  | Ensure all cables both hardwired, fibre optic and serial links connected to the above referred equipment are covered including any inter panel cables (Non-system), as necessary. |  |  |  |
|  | Ensure system wise segregation of IS, Non-IS, analog, digital, power signals to solenoids. |  |  |  |
|  | Ensure foundation field bus related design. Issues are addressed as per project specifications. |  |  |  |
|  | Check consistency of document with cable schedule, junction box schedule and vendor documents including package vendor drawings (IFC stage). |  |  |  |
|  | Check the reference drawings / documents such as – Overall control system architecture, unit plot plans, etc. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |