| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Check Border /Title block (Signatory initials, Revision Phrases, date in title block), Stamp (review/approval/construction), North direction block matches with the project drafting specification and is consistent with the deliverables from other discipline. |  |  |  |
|  | Check changes wrt old revision of drawings and client comments are indicated with proper cloud mark, & Rev. mark. |  |  |  |
|  | Check I/O list format as per Project / Client Standards / approval by lead engineer. |  |  |  |
|  | Ensure document title, revision matches with TDR. Soft file name / Path as per project specification requirement are mentioned in document. |  |  |  |
|  | Check legends, abbreviations and notes are defined properly. |  |  |  |
|  | Check notes shown on document are relevant and appropriately referenced. |  |  |  |
|  | Check no abbreviations are used in document title. Abbreviations if any are included in the legend part of the document. |  |  |  |
|  | Ensure Document Title, Rev matches with TDR. Soft file name / path as per project specific requirement are mentioned in document. Approval category to be ensured. |  |  |  |
|  | Ensure that the tags which may not be part of the P&IDs such as those belonging to F&G system, building fire alarm system, corrosion monitoring system, electrical system, AMADAS, etc. are all captured in the I/O list. |  |  |  |
|  | Ensure correctness of I/O type i.e. source type, sink type, line monitored, I.S., serial, OPC, etc. |  |  |  |
|  | Ensure the ranges units of measurement indicated are consistent with the data sheets. |  |  |  |
|  | Check alarm points are as per the P&IDs. Check that the gap control set points are included. |  |  |  |
|  | Check input obtained from the client for report generation are included or suggest the report generation envisaged as understood from the RFQ in the I/O list. |  |  |  |
|  | Ensure serial I/O addresses obtained from the required vendors of systems and package units are included in I/O lists. |  |  |  |
|  | Check notes shown on document are relevant. |  |  |  |
|  | Check holds if any are indicated on document. |  |  |  |
|  | Check all tags are covered with respect to P&ID’s. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |