| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Check Border /Title block (Signatory initials, Revision Phrases, date in title block), Stamp (review/approval/construction), North direction block matches with the project drafting specification and is consistent with the deliverables from other discipline. |  |  |  |
|  | Check changes wrt old revision of drawings and client comments are indicated with proper cloud mark, & Rev. mark. |  |  |  |
|  | Check document format as per Project / Client Standards / approval by lead engineer. |  |  |  |
|  | Check legends, abbreviations and notes are defined properly. |  |  |  |
|  | Check notes shown on document are relevant and appropriately referenced. |  |  |  |
|  | Check no abbreviations are used in document. Title. Abbreviations if any are included in the legend part of the document. |  |  |  |
|  | Ensure Document Title, Rev matches with TDR. Soft file name / path as per project specific requirement are mentioned in document. Approval category to be ensured. |  |  |  |
|  | Ensure overview of graphics represent all process units inclusive of all package units. |  |  |  |
|  | Ensure separate graphic for fire and gas detection and fire alarm system. |  |  |  |
|  | Check separate graphic for MOS and start-up override if required. |  |  |  |
|  | Check special graphics that the client has indicated in the RFQ such as shutdown page indicating status of all SDVs and BDVs. |  |  |  |
|  | Check separate graphics if required to indicate running hours of pumps, flow metering system. |  |  |  |
|  | Check separate graphics for electrical systems. |  |  |  |
|  | Ensure graphics for utility systems such as instrument air, fire water, DG set, HVAC, etc. |  |  |  |
|  | Check the flow lines on graphic are from left to right and crossing of lines is limited. |  |  |  |
|  | Check equipment sizes and symbols are representative of their actual sizes and shapes. Line color and line thickness, as predetermined, are indicated consistently in the graphics. |  |  |  |
|  | Check representation of utility lines in progress graphics not to be shown. As a practice, manual valves, safety relief valves shall not be indicated on the graphics. |  |  |  |
|  | Check continuation of lines and targets are indicated correctly between graphics. |  |  |  |
|  | Ensure the loop tag numbers are consistent with that in the I/O list and all tags including that of package systems are represented. |  |  |  |
|  | Check locations of tags are consistent with that as represented in the PID’s Engineering units as required are properly represented. |  |  |  |
|  | Check requirement of pop-up screens/help screens for complicated loops / sequences to guide the operators. Interfaces for sequence operations. |  |  |  |
|  | Check fail safe condition of valves are represented correctly in line with PIDs. |  |  |  |
|  | Ensure the format of report generation as part of the graphic drawings is available. |  |  |  |
|  | Ensure the title of the graphic is consistent with the operating unit of the plant. |  |  |  |
|  | Ensure no abbreviations are followed in document title. |  |  |  |
|  | Check all sheets are available in the set of document before deliver. |  |  |  |
|  | Check holds if any are indicated on drawing. |  |  |  |
|  | Identify the requirement of any other special requirements with respect to compressor control, trends, etc. and generate graphic scheme for system vendor’s input. |  |  |  |
|  | Check graphics for IDC to process group and the electrical graphics to the electrical group. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |