| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Check Border /Title block (Signatory initials, Revision Phrases, date in title block), Stamp (review/approval/construction), matches with the project drafting specification and is consistent with the deliverables from other discipline. |  |  |  |
|  | Check changes wrt old revision of drawings and client comments are indicated with proper cloud mark, & Rev. mark. |  |  |  |
|  | Check Line, Text properties are followed as per project drafting philosophy / CAD manual. |  |  |  |
|  | Check drawing format as per Project / Client Standards / approval by lead engineer. |  |  |  |
|  | Check legends, abbreviations and notes shown on drawing are relevant and appropriately referenced. |  |  |  |
|  | Ensure Drawing Title, Rev matches with TDR. Soft file name / path as per project specific requirement are mentioned in drawing. Approval category to be ensured. |  |  |  |
|  | Ensure all symbols are as per legend sheet. All legends as per project / client standards. |  |  |  |
|  | Ensure the legends and symbols used to represent are properly and correctly covered with clear identification and explanation. |  |  |  |
|  | Check notes shown on drawing are relevant. |  |  |  |
|  | Check holds if any are indicated on drawing. |  |  |  |
|  | Check that no abbreviations are used in Drawing title. |  |  |  |
|  | Leave spare sheets between the sections (e.g. oil, gas, Utility, Sulfur) to accommodate the future additions of units. The same shall be reflected in the Index sheet as well as within the document. |  |  |  |
|  | A sheet dedicatedly assigned for general notes related to the project explaining the higher level concepts of shutdown. E.g. ESD-1/2/3 & PSD-1/2/3. |  |  |  |
|  | A "NOTES" sheet with all the units including the equipments associated with each unit with the corresponding interlock (e.g. "UZ") block no, Unit wise Shutdown initiators and it's Resets and MOS Override tags and it's lamp (if applicable) shall be prepared. |  |  |  |
|  | Staggered Blow down Sequence and the various units associated with Blow down shall be detailed in a separate sheet. This will form part of the "NOTES" sheet. |  |  |  |
|  | In addition to the above point 5, a table of Fire zones with the relevant inter trip action generated (Confirmed fire / BGU activated / Confirmed gas) with tag numbers against the Process units (Associated equipment no’s). |  |  |  |
|  | Higher level Plant shutdown sheet shall be prepared. (Initiators creating ESD-1/PSD-1). This shall be the first C&E sheet after the "NOTES" sheet. |  |  |  |
|  | The document number for individual sheets shall have incremental sheet nos. |  |  |  |
|  | Package Unit C&E shall be integrated into the Main C&E. |  |  |  |
|  | There will be single sheet for each Unit level C&E. |  |  |  |
|  | Each Unit level C&E sheet shall contain three layers for Initiators (Cause) & Final elements (Effect) 1. Definition as First Layer : Upper level initiators (ESD-1/2/3 or PSD-1/2/3) and its effects (if applicable) 2. Inter trip signals as second layer : Signals from F&G system / between Process Units. 3. Initiators belong to the unit as 3rd Layer: All the initiators belong to that unit. The Initiators (causes) in the 3rd layer of C&E shall be prepared in the direction of process flow as defined in PFD, PSFS and PEFS i.e. Equipment and its initiators from Upstream to downstream) shall be arranged in an order. |  |  |  |
|  | In addition to the above, the Equipment along with its description as the group title below which the initiators shall be mentioned properly. |  |  |  |
|  | The description for the initiators and the Final elements mentioned in the C&E shall match the description mentioned in I/O database. |  |  |  |
|  | The initiators shall be grouped according to the Interlock (e.g. UZ) block no. |  |  |  |
|  | Information related to Voting (1oo2 or 2oo3) shall be clearly indicated against the relevant initiators. |  |  |  |
|  | The sheet shall have enough space for notes (specific information) about the initiators / final elements like timers, delayed trips etc. |  |  |  |
|  | The "X" shall be correctly marked against the relevant initiators. |  |  |  |
|  | Any change in the "X" in the next revision shall be clouded / highlighted with the Revision no marked in the specific column "REV" |  |  |  |
|  | The final elements (Effects) shall be arranged in the direction of the process flow as defined in the PFS/PSFS/PEFS and grouped with the equipment no and description as title for the group. In addition to the above the final elements of same type shall be arranged in ascending order followed by the other final elements within the same group. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |