| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Check border /Title block (Signatory initials, Revision Phrases, date in title block), Stamp (review/approval/construction), North direction block matches with the project drafting specification and is consistent with the deliverables from other discipline. |  |  |  |
|  | Ensure drawing Title, Rev matches with TDR. Soft file name/path as per project specific requirement is mentioned in drawing. Approval category to be ensured. |  |  |  |
|  | Check changes wrt old revision of Drawing and clients comments are indicated with proper cloud mark. |  |  |  |
|  | Ensure check line, text properties are followed as per project drafting philosophy / cad manual and is mentioned in the drawing. |  |  |  |
|  | Check notes shown on drawing are relevant. |  |  |  |
|  | Check holds if any are indicated on drawing with proper cloud mark. |  |  |  |
|  | Ensure that no abbreviations are used in Drg. Title. Abbreviations if any are included in the legend part of the drawing. |  |  |  |
|  | Ensure that the BOM quantity is matching with instrument installation drawing and hook-up drawings. |  |  |  |
|  | Ensure that the tray BOM is matching with the extracted MTO of 3D model. |  |  |  |
|  | Ensure that the PVC sleeves for paved area, road crossing duct banks are covered by civil and that should be captured by note in document. |  |  |  |
|  | Ensure that all tray accessories like elbows, risers, equal tees, reducers and coupler plates with bolts/nuts/washer, Etc. are covered. |  |  |  |
|  | Ensure that tray covers for all the trays wherever applicable are considered. |  |  |  |
|  | Ensure that the cable quantities are matching with issued cable schedule. |  |  |  |
|  | Ensure that the quantity for cable clamps are included. |  |  |  |
|  | Ensure that the junction box, glands and plugs quantities are matching with issued junction box schedule. |  |  |  |
|  | Ensure that the quantities of instrument tag plates are matching with issued label schedule. |  |  |  |
|  | Ensure the quantities of cable markers, core markers are included. |  |  |  |
|  | Ensure that the GI conduits, PVC shrouds are included. |  |  |  |
|  | Ensure the quantities of caution tapes, cable tiles, cable rout markers and cable ties are included. |  |  |  |
|  | Ensure that the quantities of cable core ferrules and lugs are included. |  |  |  |
|  | Check all multiple sheets are available in the set of drawing before delivery. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |