| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Is the foundation layout correctly oriented with respect to the plant North? |  |  |  |
|  | Are the reference coordinates correctly indicated as per the plot plan? |  |  |  |
|  | Are all foundations properly located with respect to the reference point? |  |  |  |
|  | Are the foundation sizes marked, as per the design document? |  |  |  |
|  | Are all the foundations checked for interference with adjacent foundations and structures? |  |  |  |
|  | Has the founding level been correctly chosen and indicated as per the soil strength requirements consistent with the soil investigation report? |  |  |  |
|  | Has the soil bearing capacity and other relevant instructions required at site during execution been correctly indicated in the "NOTES"? |  |  |  |
|  | Whether all reference documents to drawings have been indicated in the drawing? |  |  |  |
|  | Has the drawing been made as per the general guidelines for drawings and using the standard drawing formats and also as per the guidelines for preparing the foundation layouts? |  |  |  |
|  | Has the drawing been numbered as per the drawing index with the respective revision marking? |  |  |  |
|  | Whether all the latest revisions have been marked with clouds? |  |  |  |
|  | Whether the location of the relevant structure have been shaded properly in the key plan? |  |  |  |
|  | Whether spacing of piles are as per relevant codes? |  |  |  |
|  | Whether pile capacities in lateral, axial and uplift etc. are incorporated? |  |  |  |
|  | Whether the match lines are properly indicated at locations of split layouts? |  |  |  |
|  | Are Overall foundation layout of all equipment /structure/building to be prepared and updated periodically for each project whenever foundation drawing release for construction to site? |  |  |  |
|  | whether hold is marked in the area where input is awaited? |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |