| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Whether the sizes of beams/slab conform to the design document and GA drawings? |  |  |  |
|  | Are all the reinforcements indicated as per the design? |  |  |  |
|  | Are the bar spacing provided within the permissible limits as per the relevant code? |  |  |  |
|  | Are the diameter rods indicated checked with the list of available diameters of rods specified for the project? |  |  |  |
|  | Are the curtailments of main reinforcements as per the codal provision & standard detailing practice? |  |  |  |
|  | At least 1/3rd of total bottom reinforcement at mid span continues into the support for beams? |  |  |  |
|  | Is the lapping of reinforcement done near the zones of Zero moment? (i.e. at mid span for top rods and near support for bottom rods) |  |  |  |
|  | Is the % of rods lapped at any section less than 50% of rods available at the section? |  |  |  |
|  | Is the reinforcement arrangement shown in cross section match with that shown in longitudinal sections? |  |  |  |
|  | Whether the drawing is prepared as per the drafting guidelines and as per guidelines for preparing reinforcement drawings? |  |  |  |
|  | Whether all reference drawings have been indicated in the drawings? |  |  |  |
|  | Has the drawing been numbered as per the drawing index with the respective revision marking? |  |  |  |
|  | Whether all the latest revisions have been marked with clouds? |  |  |  |
|  | Whether the general notes covering the concrete grade, clear cover etc. are complete, and correct? |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |