| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Checks in Model Sketches: |  |  |  |
|  | Does the geometry of the model represent the actual structure? |  |  |  |
|  | Does the boundary conditions incorporated in the model ensure the stability of the model and conform to the actual structure? |  |  |  |
|  | Whether the marking of loads conform to load computations? a) Intensity b) Distribution c) Points of application |  |  |  |
|  | Have all the reactions from secondary system been transferred to the main frame elements properly? |  |  |  |
|  | Have all the floor levels are as per input Drawings? |  |  |  |
|  | Checks in Load Computations: |  |  |  |
|  | Whether all the basic load Cases are Covered as per the IS Code? |  |  |  |
|  | Whether the All the load values considered are as per the design basis / equipment data? |  |  |  |
|  | Are the wind Load is calculated as per variation of Height of building? |  |  |  |
|  | Are the Live load are reduced as per Floor height given in IS Code? |  |  |  |
|  | Are numerical computation of loads carried out correctly? |  |  |  |
|  | Is the distribution of loads among various members carried out correctly? |  |  |  |
|  | Are the Seismic Load Calculation are done with both Static and dynamic Method and enhancement factor is applied/calculated accordingly? |  |  |  |
|  | Checks in Input for Computer Analysis: |  |  |  |
|  | Whether the Master control is correct? |  |  |  |
|  | Whether the geometry of the model such as coordinate data and connectivity data are correct as per the validated model sketches? |  |  |  |
|  | Are the sectional properties correctly input? |  |  |  |
|  | Are the boundary conditions input correctly? |  |  |  |
|  | Have all the loads specified properly with correct intensities and points of application? |  |  |  |
|  | Has the required number of load cases and load combination been specified with Appropriate load factors? |  |  |  |
|  | Has the deflection check parameter are defined for all major members? |  |  |  |
|  | Checks in Analysis Of Secondary Beams and Systems: |  |  |  |
|  | Whether all the secondary systems are covered in the analysis? |  |  |  |
|  | Whether support conditions and support locations are chosen properly? |  |  |  |
|  | Have the loads been considered correctly as per design basis / vendor’s information? |  |  |  |
|  | Has the Analysis been done by the correct and accepted procedures? |  |  |  |
|  | Are the numerical computations correct? |  |  |  |
|  | Are the Secondary Members are lesser sizes w.r.t Connecting/Primary Member? |  |  |  |
|  | Checks in Analysis Of Output: |  |  |  |
|  | Is the total externally applied load as calculated matching with the sum of support of reactions obtained from the output? |  |  |  |
|  | Are the output values of various parameters consistent with the Input data? |  |  |  |
|  | Are there any abnormal values of parameters such as nodal deflections or member forces which are not consistent with the structural configuration and loading? |  |  |  |
|  | All The Warning shall not be Major? |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |