| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Checks in Design: |  |  |  |
|  | Check complete facility is shown (as per the scope of work) |  |  |  |
|  | Check plant north, true north, wind direction & wind rose (if applicable) is shown |  |  |  |
|  | Check Security wall and Fencing around the plant, Main gates with Security room/Guard house and emergency exits are shown |  |  |  |
|  | Check main roads and intermediate roads are located with centerline coordinates/dimensions and road width is given. Check the turning radius of the roads are shown as per standard |  |  |  |
|  | Check major distances between units/equipment are given |  |  |  |
|  | Check all grade levels are given |  |  |  |
|  | Check all areas are indicated by names/tags for e.g. Utility area, Compressor area etc. |  |  |  |
|  | Check minimum distance requirement as per the applicable codes/Client standards are indicated (At least for IFA stage) |  |  |  |
|  | When plant (relative) co-ordinate system is used, the N-E corner of the plot shall be the datum point. Check that the true geographical co-ordinates (UTM) of datum are indicated in the notes. |  |  |  |
|  | Check sterile zone radius is shown for flare stack (if applicable) |  |  |  |
|  | Check reference with existing facilities is given (if applicable) |  |  |  |
|  | Check pipe rack / pipe way is shown (without dimensions) |  |  |  |
|  | Check Hold list (if any) |  |  |  |
|  | Check co-ordinate grids are properly shown where applicable |  |  |  |
|  | Check all future requirement (e.g. future units/equipment, expansion area etc.) is provided and clearly marked |  |  |  |
|  | Check that tag nos. of equipment are shown on the plot plan (only if required by client), If equipment list is given, check that all tagged equipment is shown in the plot plan and match the tag numbers with equipment list/PFD |  |  |  |
|  | Checks in Drafting: |  |  |  |
|  | Check all tags are clearly marked and it is legible |  |  |  |
|  | Check all coordinates/dimensions are as per the Cad standard/Project standard |  |  |  |
|  | Check title block details, revision no. and Drawing no. as per the TDR. Check that revision block contains initials of the originator, checker and approver (both Discipline & Project) |  |  |  |
|  | Check all general notes. |  |  |  |
|  | Check Graphic Scale (Bar scale) is shown |  |  |  |
|  | Check Location map/Key plan is shown (if applicable) |  |  |  |
|  | Check Legend is shown (for e.g. Showing scope cloud, new/existing facilities etc.) |  |  |  |
|  | Check all reference Drawing numbers. |  |  |  |
|  | Check the units for dimensions, co-ordinates & levels are given. |  |  |  |

Notes:

- Check List must be filled up for each issue of plot plan.

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |