| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Checks in Design: |  |  |  |
|  | Plant North and True North are shown. |  |  |  |
|  | Grade and paving levels are shown. |  |  |  |
|  | Extents of paving areas are shown with curbing and drain sump, as applicable (dimensions not required). |  |  |  |
|  | Unit/area code, name or tag (e.g. UTA, Utility Area, Unit-109, etc.) is indicated. |  |  |  |
|  | Main roads and secondary roads are located with center line and coordinates/locating dimensions. |  |  |  |
|  | Road width and elevations are given. |  |  |  |
|  | Road crossings, e.g. culverts, casing pipe, duct banks are shown wherever applicable. |  |  |  |
|  | Crash barriers are shown wherever required. |  |  |  |
|  | Emergency exits are shown. |  |  |  |
|  | Availability of access to heavy equipment for erection purpose is checked. |  |  |  |
|  | Hard stand areas for heavy equipment is properly marked and located (if applicable). |  |  |  |
|  | Equipment list is given in the drawing and equipment tags marked in the plot match with the list. |  |  |  |
|  | Equipment/packages and their tag nos. are checked with the Project Equipment List/P&IDs. |  |  |  |
|  | All equipment/packages are shown with locating coordinates/dimensions and elevations (centerline, BTL, u/s of base plate, etc.) and the elevations are checked wrt P&ID/Process requirements. |  |  |  |
|  | All equipment sizes are checked with mechanical data sheets/Vendor drawings. |  |  |  |
|  | Auxiliary equipment (e.g. API seal plans, LO coolers/skids, etc.) of pumps/compressors and other packages are shown as per Vendor data. |  |  |  |
|  | Multiple equipment having tag nos. with suffixes like A/B/C (e.g. multi-bay air-coolers, pumps, etc.) are clearly identified and tagged as per Project Equipment List/P&IDs. |  |  |  |
|  | Fixed/sliding saddle locations for horizontal vessels and heat exchangers are indicated and the direction of slope is indicated for sloping vessels. |  |  |  |
|  | Pipe racks are shown with the module breaks and tag nos. (As per Civil drawings) and the locating coordinates, column to column dimensions and widths are indicated. |  |  |  |
|  | Pipe rack TOS elevations are clearly marked, esp. in case of multi-tier racks. Sectional details are provided wherever required and space requirement for cable trays is marked. |  |  |  |
|  | Pipe tracks/sleepers are shown with locating coordinates/dimensions, widths, TOS elevations and tag numbers (if applicable). |  |  |  |
|  | Minimum distances between equipment/buildings/units as per the applicable codes/Project standards are checked. |  |  |  |
|  | Laydown/dropdown areas, exchanger tube removal areas, equipment maintenance/crane access areas are shown wherever required. |  |  |  |
|  | The requirement of monorail/EOT/HOT cranes/pipe davits, if any, for compressor shelters/pump houses/platforms, etc. is shown with elevation & capacity. |  |  |  |
|  | Davit location/orientation on heavy equipment/columns/heat exchangers, etc. is checked for access/maintenance handling. |  |  |  |
|  | Sufficient access clearance for fork lift/crane/vehicle is available within the unit area. |  |  |  |
|  | All platforms are shown and elevations (TOG) indicated (locating dimensions not required). |  |  |  |
|  | All buildings/shelters/technological structures (including analyzer shelters, if applicable) are located with coordinates & tag numbers and overall dimensions are given. |  |  |  |
|  | Floor plans/sections of shelters/technological structures are shown where necessary. Equipment and platforms within shelters are shown. |  |  |  |
|  | Structural grid numbers of pipe racks, buildings and shelters are shown. Grids numbers are marked with numerals in one direction and alphabets in the other direction as per the project requirements. |  |  |  |
|  | Future requirements (e.g. future units/equipment, expansion areas, etc.) are provided and clearly marked. |  |  |  |
|  | Underground facilities (e.g. buried vessels, u/g storage pits, etc.) are shown. Items like valve pits, OWS manholes, etc. are shown in model extracted drawings (dimensions/coordinates are not required). |  |  |  |
|  | Storm water drain trenches and major UG corridors (for UG piping, pipelines, cable trenches, etc.) are shown (dimensions/ coordinates are not required). Foundations below ground need not be shown. |  |  |  |
|  | HSE requirements (if any) for staircase/escape route for tech. structure and pipe rack platforms are checked. |  |  |  |
|  | Sterile zone radius is shown for flare stack and no other equipment is located within the sterile zone. |  |  |  |
|  | Safety distance/radius is shown for fired heater/hazardous equipment (if any). |  |  |  |
|  | HSE facilities/equipment like deluge valve skids, foam skids, safety showers, eye washes, wind socks, assembly points, etc., are shown, as applicable. |  |  |  |
|  | Dyke walls are shown around tanks (wherever applicable) with dimensions and heights. |  |  |  |
|  | Dyke walls are shown around tanks (wherever applicable) with dimensions and heights. |  |  |  |
|  | Volume bottles for SDV s are shown on platforms/modules, wherever applicable. |  |  |  |
|  | The drawing border conforms to the Project specifications or Company standards, as applicable. |  |  |  |
|  | Drafting conventions e.g. font types/sizes, text styles, line types, coordinate/dimension styles, layer structure, etc. are as per the Project specifications or Company CAD standards, as applicable. |  |  |  |
|  | Drawing title, drawing number and revision number in the title block are as per the TDR. |  |  |  |
|  | Electronic file name is indicated at appropriate location on the drawing. |  |  |  |
|  | Revision block contains initials of the originator, checker and approver (both Discipline & Project). |  |  |  |
|  | Drawing scale is stated in the title block and graphic scale (bar scale) is shown. |  |  |  |
|  | The drawing scale conforms to the Project specifications or Company standards, as applicable. |  |  |  |
|  | All general notes are checked and the units for dimensions, coordinates & levels are given in the notes. |  |  |  |
|  | Legend for applicable symbols is shown (e.g. new/existing facilities, scope cloud, etc.) |  |  |  |
|  | Reference drawing numbers and titles are listed and checked. List shall include overall plot plan and key plot plan as a minimum. |  |  |  |
|  | Status stamp (e.g. For Approval, For Construction) is shown. |  |  |  |
|  | Revision clouds, along with revision number triangles are marked properly, if applicable. |  |  |  |
|  | Hold clouds, if applicable, are properly marked and the list of Holds is indicated. |  |  |  |
|  | All tags are clearly marked and are legible. |  |  |  |
|  | Match line co-ordinates and drawing limits are clearly marked and are consistent with the Key Plan drawing. Continuation drawing numbers are correctly written. |  |  |  |
|  | Key plan is shown and the applicable area/unit of the plot plan is highlighted. |  |  |  |
|  | All tags are clearly marked and are legible |  |  |  |
|  | Drawing extracted from 3D CAD model is not manually edited. |  |  |  |
|  | Checks in Drafting: |  |  |  |
|  | The drawing border conforms to the Project specifications or Company standards, as applicable. |  |  |  |
|  | Drafting conventions e.g. font types/sizes, text styles, line types, coordinate/dimension styles, layer structure, etc. are as per the Project specifications or Company CAD standards, as applicable. |  |  |  |
|  | Drawing title, drawing number and revision number in the title block are as per the TDR. |  |  |  |
|  | Electronic file name is indicated at appropriate location on the drawing. |  |  |  |
|  | Revision block contains initials of the originator, checker and approver (both Discipline & Project). |  |  |  |
|  | Drawing scale is stated in the title block and graphic scale (bar scale) is shown. |  |  |  |
|  | The drawing scale conforms to the Project specifications or Company standards, as applicable. |  |  |  |
|  | All general notes are checked and the units for dimensions, coordinates & levels are given in the notes. |  |  |  |
|  | Legend for applicable symbols is shown (e.g. new/existing facilities, scope cloud, etc.) |  |  |  |
|  | Reference drawing numbers and titles are listed and checked. List shall include overall plot plan and key plot plan as a minimum. |  |  |  |
|  | Status stamp (e.g. For Approval, For Construction) is shown. |  |  |  |
|  | Revision clouds, along with revision number triangles are marked properly, if applicable. |  |  |  |
|  | Hold clouds, if applicable, are properly marked and the list of Holds is indicated. |  |  |  |
|  | Key plan is shown and the applicable area/unit of the plot plan is highlighted. |  |  |  |
|  | All tags are clearly marked and are legible. |  |  |  |
|  | Match line co-ordinates and drawing limits are clearly marked and are consistent with the Key Plan drawing. Continuation drawing numbers are correctly written. |  |  |  |
|  | Drawing extracted from 3D CAD model is not manually edited. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |