| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Checks for Design: |  |  |  |
|  | Check with IFC P&ID: |  |  |  |
|  | 1.1 Check Line number and pipe class as per P&ID |  |  |  |
|  | 1.2 Check Equipment, Valve and Instrument tag nos. as per P&ID |  |  |  |
|  | 1.3 Check Flow direction as per P&ID |  |  |  |
|  | 1.4 Check Slope shown as per P&ID |  |  |  |
|  | 1.5 Check Minimum & Maximum distances maintained as specified in P&ID |  |  |  |
|  | 1.6 Check all special requirement covered in Notes in P&ID |  |  |  |
|  | 1.7 Check NO POCKET requirement is maintained. |  |  |  |
|  | Check all equipment is located as per the unit plot plan and N-E coordinates are given. |  |  |  |
|  | Check pipe spacing between pipes (mainly on pipe rack/sleepers) |  |  |  |
|  | Check all lines are identified and locating dimensions & elevation are given. In general pipe elevations shall be referred to BOP. |  |  |  |
|  | Check all pipe supports are marked and tagged |  |  |  |
|  | Check all equipment nozzles are located and tagged with size, rating and elevations (either showing on nozzle or providing nozzle schedule table) |  |  |  |
|  | Check all platforms and ladder/staircase are located with dimensions and elevations are marked. |  |  |  |
|  | Check all primary support symbols are indicated. (E.g. symbol of guide, axial stop etc.) |  |  |  |
|  | Check any reserved space is clearly marked. (E.g. space for heat exchanger bundle removal, laydown area etc.) |  |  |  |
|  | Check support cleat location and details for vessel platforms |  |  |  |
|  | Check flange joints of removable spools and ensure that free access for removable spools is provided |  |  |  |
|  | Check all in-line and on-line instruments/items are located |  |  |  |
|  | Check sufficient clear space all around and vertical for Corrosion Coupon and Corrosion Probe is provided (min 1m dia from centre around and 3m vertical for retriever kit) |  |  |  |
|  | Check min. spool length between 2 welds is as per project specification. |  |  |  |
|  | Check sufficient space for Axial stop installation (mainly at PSV outlet line, between Control valve and Isolation valve etc.) |  |  |  |
|  | For supports taken from ground check civil pedestal is located (e.g. for ‘T’ type support supporting many lines, civil foundation to be located) |  |  |  |
|  | Check for interference of cable tray, HVAC ducting |  |  |  |
|  | Check clear access for Control valves, SDV’s, and PSV’s are provided, this includes vertical clearance for control valve removal |  |  |  |
|  | Check for interference with Civil/Structural dwgs. |  |  |  |
|  | Check stress analysis comments are incorporated. |  |  |  |
|  | Check all IDC comments are incorporated or resolved with originator of comments. |  |  |  |
|  | Check minimum headroom clearance (2500mm preferably) is maintained in access areas. |  |  |  |
|  | Check all HOLD items are identified and hold list is indicated. |  |  |  |
|  | Check Vent and Drain for lines where Isometrics are not made (Pipe rack) are provided. Preferably at same location on pipe rack. |  |  |  |
|  | Check accessibility for SDV, ESDV & Control valves removal for hydro testing (Mainly large sizes) |  |  |  |
|  | Check all valves hand wheels and actuators are shown with correct orientation. |  |  |  |
|  | Check volume bottles for SDV’s and ESDV’s are located. |  |  |  |
|  | Check all Tie-ins for Construction/HSE philosophy or special Tie-in procedure if available |  |  |  |
|  | Check the size of civil foundation/insert plate for guide support |  |  |  |
|  | Check emergency escape route/access are provided as per HSE requirement. |  |  |  |
|  | Check welded shoe/trunnion supports are not provided on Equal Tee/Red. Tee. |  |  |  |
|  | Check basket/screen removal space is available for pump suction strainers. |  |  |  |
|  | Check that removable spools are provided to facilitate equipment maintenance/removal. |  |  |  |
|  | Check Upstream/Downstream straight pipe requirements at butterfly valves, control valves, check valves, flow elements (orifice), flow metres, and pump suction & discharge nozzles etc are satisfied. (The upstream/downstream requirements may not be indicated in P&ID.) |  |  |  |
|  | Check PSV outlet lines are self draining to flare header. |  |  |  |
|  | Check two-phase lines are identified and appropriately designed. |  |  |  |
|  | Check that lines have the flexibility to install blinds/spacers. |  |  |  |
|  | Check access for valves, instruments, man ways, inspection ports etc, as per project specification, ensure that equipment removal path/fork lift access aisles are considered in the layout. |  |  |  |
|  | Checks for Drafting: |  |  |  |
|  | Check match line co-ordinates and dwg no. is given |  |  |  |
|  | Check all equipment tags are shown as per the drafting standard. |  |  |  |
|  | Check all line numbers, elevations, platform levels etc are shown as per the drafting standard. |  |  |  |
|  | Check all lines 2” and above are shown with double line (Unless specified otherwise in project design basis) |  |  |  |
|  | Check all new lines in existing areas/drawings are identified with either in clouds or bold lines depending on project philosophy/client standard. |  |  |  |
|  | Check Key plan is shown and relevant drawing area is highlighted |  |  |  |
|  | Check all notes, this includes notes for units of dimensions/elevations, reference of true coordinates with plant co-ordinates etc. |  |  |  |
|  | Check legend is shown |  |  |  |
|  | Check scale bar (bar scale) is shown |  |  |  |
|  | Check title block details, revision no. and dwg no. as per TDR. Check that revision block contains initials of the originator, checker and approver (both discipline & project) |  |  |  |
|  | Check plant north and geographical north arrows are marked. |  |  |  |
|  | Check line types for centerline, match line, U/G piping is correctly defined as per cad standard |  |  |  |
|  | Check dimension style is as per cad standard (i.e. dimensions text height, style, arrow block etc) |  |  |  |
|  | Check status stamp is shown (e.g. For Approval, For Construction etc) |  |  |  |
|  | Check Revision clouds are marked properly along with revision no. triangle. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |