| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Checks for Design: |  |  |  |
|  | Check Plant North & True North are shown |  |  |  |
|  | Check all equipment, Pipe sleepers, Pipe Racks are shown and located |  |  |  |
|  | Check Paving area is shown with curbing & drain sump (if applicable) |  |  |  |
|  | Check all equipment are tagged correctly |  |  |  |
|  | Check multiple equipment of tag nos. with prefixes like A, B, C, are clearly identified and tagged accordingly (mainly for multibay air-coolers, Pumps etc.) |  |  |  |
|  | Check all areas are indicated by names/Tags for e.g... Utility area, Compressor area etc. |  |  |  |
|  | Check minimum distance requirement as per the applicable codes/client standards |  |  |  |
|  | Check specific requirement of lay down area, space for exchanger tube removal & maintenance access areas. |  |  |  |
|  | Check for heavy equipment erection access and Monorail requirement |  |  |  |
|  | Check all platforms are shown (Only for information, not required to locate) |  |  |  |
|  | Check Volume Bottles for SDV's are located |  |  |  |
|  | Check all buildings are located with coordinates (at least one corner) |  |  |  |
|  | Check Grade levels/Paving levels are shown. |  |  |  |
|  | Check any future space requirement is provided and clearly marked |  |  |  |
|  | Check whether process flow is maintained. |  |  |  |
|  | Check the equipment list on unit plot plan with tag nos. marked on unit plot plan. Check the equipment tag no. & description (name) with PFD/Equipment list. |  |  |  |
|  | Check Hold list (if any) |  |  |  |
|  | Check all Roads are located and Road width is given |  |  |  |
|  | Check fixed saddle location for horizontal vessels is shown. |  |  |  |
|  | Check underground facility (e.g. valve pits, u/g storage pits, buried vessels etc.) is shown |  |  |  |
|  | Check all equipment and major platforms elevations are given |  |  |  |
|  | Check all equipment with data sheets/Vendor drawings. |  |  |  |
|  | Check pipe rack future space is provided (if applicable) |  |  |  |
|  | Check the crane/vehicle access at pipe rack road crossing. |  |  |  |
|  | Check that section/elevations of pipe racks are given. |  |  |  |
|  | Check structural grid numbers of Pipe Racks, Buildings and Shelters are shown. Grids shall be arranged ascending numerals from left to right (West to East) and alphabets from bottom to top (South to North) |  |  |  |
|  | Check Pipe Sleepers are shown with tag nos. |  |  |  |
|  | Check that section/elevations of pipe racks are given. |  |  |  |
|  | Checks for Drafting: |  |  |  |
|  | Check all Tags are clearly marked and it's Legible |  |  |  |
|  | Check all coordinates/dimensions are as per the Cad standard/project standard |  |  |  |
|  | Check Match line co-ordinates and Drawing limits. |  |  |  |
|  | Check Title block details, Revision no. and Dwg no as per the TDR. Check that revision block contains initials of the originator, checker and approver (both Discipline & Project). |  |  |  |
|  | Check all general notes. |  |  |  |
|  | Check Graphic Scale (Bar scale) is shown |  |  |  |
|  | Check Key plan is shown and the area/unit of the plot plan is highlighted. |  |  |  |
|  | Check Legend is shown (for e.g. Showing scope cloud, new/existing facilities etc. |  |  |  |
|  | Check all reference dwg nos. (minimum overall plot plan and key plan) |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |