| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Checks for Design: |  |  |  |
|  | Following items shall be checked with IFC P&ID: - Check Line number & Pipe class as per P&ID. - Check Equipment, Valve, SP item and Instrument tag nos. as per P&ID - Check flow direction is marked as per P&ID - Check slope shown as per P&ID - Check minimum & maximum distances/Equipment Elevations are maintained as specified in P&ID - Check Spectacle Blind or Spacer & Blind Open/Close position as per P&ID - Check all special requirements given in P&ID Notes are taken care. - Check all components located in the line and the branch connections are in the same sequence as per P&ID - Check that spec break is marked as per P&ID - Check special feature of valves such as LO, LC, FB etc. are shown in isometrics as per P&ID |  |  |  |
|  | Ensure gaskets and bolts of higher class/material are reported at flange where spec break is provided |  |  |  |
|  | Check dimensions with IFC Piping GA drawing |  |  |  |
|  | Check instrument connections with Piping-Instrumentation interface drawings and Instrument hook-up drawings |  |  |  |
|  | Check all tagged instrument items with respect to approved vendor drawings |  |  |  |
|  | Check that Orifice tapping orientation is as per Instrument standard for gas and liquid lines |  |  |  |
|  | Check F/F dimensions all valves and SP Items with respect to approved vendor drawings |  |  |  |
|  | Check design, operating and testing parameters with IFC line list |  |  |  |
|  | Check Insulation/ Painting requirements, PWHT and Radiography requirements are indicated as per IFC line list. |  |  |  |
|  | Check the following have been specified in the isometrics as appropriate: - Insulation limit e.g. for PP insulation or as per P&ID - Electrical tracing and limits of the same - Wrapping and Coating limits |  |  |  |
|  | Check high point vents and low point drains have been provided for lines that will be hydro tested. (e.g. IA, Flare, N2 lines may not be hydro tested, but pneumatically tested) |  |  |  |
|  | Check appropriate primary supports are selected based on selection chart, e.g., for SS lines, Alloy steel, sloping lines, vibrating lines. Ensure all attributes for a type of support are complete |  |  |  |
|  | Check Equipment/ Package nozzle no., size, rating, orientation and distance from centre of the equipment w.r.t. approved vendor drawings |  |  |  |
|  | Check required space availability for installing axial stop and guide supports |  |  |  |
|  | Check Upstream and Downstream straight length for orifice flanges, Flow meters, Butterfly valves, Check valves etc. |  |  |  |
|  | Check straight length requirement for suction & discharge piping for pumps as required by Vendor |  |  |  |
|  | Check isometrics with respect to constructability, process requirement & special operational philosophy (mainly at Tie-in work) |  |  |  |
|  | Check Aboveground / Belowground demarcation is shown where applicable |  |  |  |
|  | Check penetration through platforms, dykes, building walls and floors has been shown |  |  |  |
|  | Check requirement of reinforcement pad for welded branch connection as per Branch table or calculations. |  |  |  |
|  | For galvanized threaded piping, check unions are provided at threaded terminal points and at other places where in-line items need to be removed for maintenance. |  |  |  |
|  | Check there is no interference due to expansion of piping in hot condition. |  |  |  |
|  | Check that shoe supports on pipe rack will be effective in hot condition. (Shoe length may be increased or may be offset from center of beams to ensure supports do not jump off the beams) |  |  |  |
|  | Check that lines connected to heat exchangers and pumps can be disconnected for removal of complete exchanger / pump for maintenance. Where natural breaks do not exist discuss with Layout engineer |  |  |  |
|  | Ensure minimum distance between welds is 50mm for wall thickness < 9.53 mm and six (6) times wall thickness for larger thickness unless indicated otherwise in Project specifications |  |  |  |
|  | Check that Flange with jack screws are used for spectacle blinds and listed in BOM as unique item. In the absence of Project specification, following guidelines may be used: Class 150 & Class 300 : 6” & above pipe size Class 600, Class 900 : 2” & above pipe size & Class 1500 |  |  |  |
|  | Checks for MTO: |  |  |  |
|  | Check MTO includes material for trunnion pipes |  |  |  |
|  | Check material specs are selected correctly where spec break is marked and inline items of different rating are used |  |  |  |
|  | Check extra Bolt length for PSV inlet, Wafer Check Valves, Butterfly Valves, Spectacle Blinds, Spade and Spacers and special items mounted between flanges such as Conical Strainer, Drip Rings, Insulating Gaskets etc. |  |  |  |
|  | Check full coupling/ Unions are considered where applicable (i.e. small-bore piping) |  |  |  |
|  | Check correct length of the M/C bolts for GRE flanges and for GRE / CS spec break flange assembly. |  |  |  |
|  | Check that correct item codes are used for unique items such as flange with jack screw, valves with locking arrangement, etc. For example, higher rating flange with thickness matching the lower rating pipe class, which is required at spec break, will have a unique item code. |  |  |  |
|  | Checks for Drafting: |  |  |  |
|  | Check all Olet’s, Tee's and Insulating Gaskets are called out |  |  |  |
|  | Check all Special Items are called out |  |  |  |
|  | Check special feature of valves such as LO, LC, FB etc |  |  |  |
|  | Check out-of-spec items are marked properly |  |  |  |
|  | Check Flow arrow, continuation Iso no. and Reference Dimensions |  |  |  |
|  | Check equipment nozzle numbers are called out |  |  |  |
|  | Check orientation of Valve Hand wheel’s/Actuators are shown |  |  |  |
|  | Check Title block, revision block and General notes |  |  |  |
|  | Check P&ID No. and Piping G.A. no is correctly indicated |  |  |  |
|  | Check Stress Analysis system number is mentioned in the iso. |  |  |  |
|  | Check that flow direction for items such as check valves, globe valves, strainers etc. is clearly marked. |  |  |  |

Notes:

- This isometric checklist is issued for the guidance of the isometric checkers and designers. The Lead Engineer / Area Engineer shall customize the checklist in accordance with the particular requirements as dictated by Project specifications and Piping design basis of the specific Project.

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- The isometric check list shall be finalized at the start of the design work as this checklist is intended to serve as guidelines for the design & drafting team.

- Documents to be used for checking isometrics:

- Issued For Construction P&IDs and Line lists

- Approved for Fabrication (or As-Built) Equipment & Package mechanical drawings

- Issued For Construction Instrument Hook-up and Interface drawings

- Approved drawings of in-line instrument items including Control Valves, Pressure Safety Valves, Motor Operated Valves and Emergency Shut Down Valves

- Approved SP item vendor drawings and manual Valve vendor drawings

- Approved Pipe support specification and Piping Material specification

- Issued For Construction Civil and Structural drawings wherever applicable

- In case approved drawings are not available at the time of isometrics issue, the affected items and pipe spools shall be clearly identified as “HOLD” items. In case of any doubt, the “HOLD” issues should be brought to the notice of the Area Engineer / Lead engineer for resolution.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |