| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Checks for Design: |  |  |  |
|  | Following items shall be checked with IFC P&ID: - Check Line number & Pipe class as per P&ID. - Check Equipment, Valve, SP item and Instrument tag nos. as per P&ID - Check flow direction is marked as per P&ID - Check slope shown as per P&ID - Check minimum & maximum distances/Equipment Elevations are maintained as specified in P&ID - Check Spectacle Blind or Spacer & Blind Open/Close position as per P&ID - Check all special requirements given in P&ID Notes are taken care. - Check all components located in the line and the branch connections are in the same sequence as per P&ID - Check that spec break is marked as per P&ID - Check special feature of valves such as LO, LC, FB etc. are shown in isometrics as per P&ID |  |  |  |
|  | Ensure gaskets and bolts of higher class/material are reported at flange where spec break is provided. |  |  |  |
|  | Check that isometric number is complete in the specified format and with the area suffix. |  |  |  |
|  | Ensure that change in isometric number takes place only at a flange joint/valve and not at a weld joint as far as possible |  |  |  |
|  | Check that lines connected to heat exchangers and pumps can be disconnected for removal of complete exchanger / pump for maintenance. Where natural breaks do not exist discuss with Layout engineer |  |  |  |
|  | Check high point vents and low point drains have been provided for lines that will be hydro tested. (e.g. IA, Flare, N2 lines may not be hydro tested, but pneumatically tested). |  |  |  |
|  | Check all IFC Line list parameters are correctly shown in the isometric. This should preferably done as system check. |  |  |  |
|  | Check PWHT requirement is indicated wherever applicable. |  |  |  |
|  | Check the following have been specified in the isometrics as appropriate: - Insulation limit as per P&ID (eg. PP insulation) - Electrical tracing and limits of the same - Wrapping and Coating limits. |  |  |  |
|  | Where weldolets, sockolet, half coupling, stub-in Tees are used for branch connections, ensure adequate space exists between weld edges. (In PDMS, system only checks the spool from centre of such components to the start point of next component). |  |  |  |
|  | Ensure minimum distance between welds is 50mm for wall thickness < 9.53 mm and six (6) times wall thickness for larger thickness unless indicated otherwise in Project specifications. |  |  |  |
|  | Check requirement of reinforcement pad for welded branch connection as per Branch table or calculations. |  |  |  |
|  | Check that Flange with jack screws are used for spectacle blinds and listed in BOM as unique item. In the absence of Project specification, following guidelines may be used: Class 150 & Class 300 : 6” & above pipe size Class 600, Class 900 : 2” & above pipe size & Class 1500 |  |  |  |
|  | Check lines running towards unit pipe rack from within the unit are maintained at minimum TOS EL of 2500 from grade to the extent possible. |  |  |  |
|  | Check that all vertical lines rising or dropping from rack to unit are maintained at the same back of pipe dimension from rack. |  |  |  |
|  | Check dyke penetration/platform penetration/grade penetration/ building wall / floor penetration has been shown. |  |  |  |
|  | Check that maintenance access is available for Control valves, ESDV, PSV and Actuators of valves. |  |  |  |
|  | Check that flow direction for items such as check valves, globe valves, strainers etc. is clearly marked. |  |  |  |
|  | Where feasible, provide slope in lines where 2-phase flow is expected. |  |  |  |
|  | Check piping interface with Equipment / Package nozzles is correct w.r.t. size, rating and face finish. Check that Equipment / Package drawings generated from draft mode are checked with approved vendor drawings. Check prints required. |  |  |  |
|  | For galvanized threaded piping, check unions are provided at threaded terminal points and at other places where in-line items need to be removed for maintenance. |  |  |  |
|  | Check low point drains are provided for draining the liquids in removable spools. |  |  |  |
|  | Check low point drains with liquid traps, wherever required, are provided in Plant air lines. |  |  |  |
|  | Check F/F dimensions all valves and SP Items with respect to approved vendor drawings. |  |  |  |
|  | Check instrument connections with Piping-Instrumentation interface drawings and Instrument hook-up drawings. |  |  |  |
|  | Check all tagged instrument items F/F dimensions with respect to approved vendor drawings |  |  |  |
|  | Check that Rotameter are installed in vertical with flow in upward direction (if applicable). |  |  |  |
|  | Check that isolation valve for pressure gauge is one side SW, other side Threaded. Where PI/PT tapping is taken on vertical part of line, check with instrument hook-up drawing for details. |  |  |  |
|  | Check that Orifice tapping orientation is as per Instrument standard for gas and liquid lines. |  |  |  |
|  | Check that clearance and space required for in-line instrument items such as Control valve actuator, mass flow meter etc. is available. |  |  |  |
|  | Check upstream and downstream straight length requirement has been provided for Orifice flanges, Flow meters, Butterfly valves, Check valves, etc. as per Project standards / vendor requirements. |  |  |  |
|  | Ensure portion of line containing a magnetic/Vortex/turbine flow meter is always flooded (if applicable). |  |  |  |
|  | Check straight length requirement for suction & discharge piping for certain pumps as required by Vendor. |  |  |  |
|  | Check appropriate primary supports are selected based on selection chart, e.g., for SS lines, Alloy steel, sloping lines, vibrating lines. Ensure all attributes for a type of support are complete. |  |  |  |
|  | Secondary supports should be serially numbered, attached to appropriate line and attributes should be completed. Distinction between standard support “PS” and civil support “CS” should be made while placing support |  |  |  |
|  | Check that support span does not exceed that specified in the support standard. Span chart for appropriate material and temperature range shall be used for this check |  |  |  |
|  | Check that TOC of civil supports taken from ground/paving are maintained at minimum 150 mm above HPP of paving unless specified otherwise in project design basis. |  |  |  |
|  | Check that SS pipes are not resting directly on steel. Stainless steel bearing plate (BP) or other suitable support shall be provided as per support specification. |  |  |  |
|  | Clamp type shoe to be provided for pipes requiring PWHT. Clamp type shoes shall also be used for Stainless steel, Alloy steel and galvanized pipes. |  |  |  |
|  | Ensure that non-standard guide for the Guides and axial stops is specifically called as shown in examples below: |  |  |  |
|  | (a) In case of Guide |  |  |  |
|  | Normal gap G1-10” |  |  |  |
|  | Specific gap of 6 mm G1-10” with gap 6mm |  |  |  |
|  | (b) In case of 3-way restraint |  |  |  |
|  | Normal gap G1-10” & LS1-10” |  |  |  |
|  | Specific gap of 6 mm G1-10” with gap 6mm and LS1-10” with gap 6mm |  |  |  |
|  | Check there is no interference due to expansion of piping in hot condition. |  |  |  |
|  | Check that the shoe supports on pipe rack will be effective in hot condition. (Shoe length may be increased or shoes may be offset from centre of beams to ensure that supports do not jump off the beams.) |  |  |  |
|  | Checks for MTO: |  |  |  |
|  | Check MTO includes material for trunnion pipes |  |  |  |
|  | Check material specs are selected correctly where spec break is marked and inline items of different rating are used |  |  |  |
|  | Check extra Bolt length for PSV inlet, Wafer Check Valves, Butterfly Valves, Spectacle Blinds, Spade and Spacers and special items mounted between flanges such as Conical Strainer, Drip Rings, Insulating Gaskets etc. |  |  |  |
|  | Check full coupling/ Unions are considered where applicable (i.e. small-bore piping) |  |  |  |
|  | Check correct length of the M/C bolts for GRE flanges and for GRE / CS spec break flange assembly. |  |  |  |
|  | Check that correct item codes are used for unique items such as flange with jack screw, valves with locking arrangement, etc. For example, higher rating flange with thickness matching the lower rating pipe class, which is required at spec break, will have a unique item code. |  |  |  |
|  | Additional Checks for JACKET LINES: |  |  |  |
|  | Line number and sheet number for both Jacket and core pipes should match. |  |  |  |
|  | The steam inlet for the Jacket shall be at the highest point (or on top of pipe) and condensate return must be at the lowest point or Bottom of pipe. |  |  |  |
|  | Check that the maximum span between two flanges is as per project specification and the total length of the steam circuit does not exceed the maximum specified in specification. |  |  |  |
|  | The liquid Sulphur lines shall have crosses instead of elbows or tees. |  |  |  |
|  | Special attention to be taken for the tapping’s, elbows (for gas lines) and reducers on the core lines for avoiding clash with jacket pipe. |  |  |  |
|  | Additional Checks before issue of Isometrics: |  |  |  |
|  | Check that line number matches with Isometric number |  |  |  |
|  | Check that the Pipe class printed at the bottom of the Isometric matches with that in the line number and that Insulation Spec, SA System No., Paint code, Inch Dia are mentioned at the bottom of isometric. |  |  |  |
|  | Check that continuation notes appear on each sheet of a multi-sheet isometric |  |  |  |
|  | Check that each item of graphic in an isometric is identified with a numbered box to ensure it is reported on the MTO side of Iso |  |  |  |
|  | Check two sets of stud bolts are reported for a flanged item, and one set for a wafer type item. (Except lug type wafer check valves with tapped holes for which two sets of stud bolts are required). |  |  |  |
|  | Check that no item description on MTO side is blank, and for the same description and size, the item is listed with a unique item number. |  |  |  |
|  | Check that all HOLDS are clearly identified with cloud or included in notes on isometric |  |  |  |
|  | Check equipment nozzle numbers are called out |  |  |  |
|  | Ensure that the isometric prints are readable. If required, the isometric should be split into more sheets. |  |  |  |
|  | Check adequacy of distance between support and the adjacent component (e.g. between shoe support and elbow / flange). In PDMS, it is not checked by system though it is clearly seen on screen. |  |  |  |
|  | Check co-ordinates of the start and end point of a Tee (branch connection) on the header isometric and the branch isometric are matching, where a new isometric starts from Tee branch. |  |  |  |
|  | Check no out-of-spec component exists in isometric. (This check is to be carried out by 3D designer). |  |  |  |

Notes:

- This isometric checklist is issued for the guidance of the isometric checkers and designers. The Lead Engineer / Area Engineer shall customize the checklist in accordance with the particular requirements as dictated by Project specifications and Piping design basis of the specific Project.

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- The isometric check list shall be finalized at the start of the design work as this checklist is intended to serve as guidelines for the design & drafting team.

- Documents to be used for checking isometrics:

- Issued For Construction P&IDs and Line lists

- Approved for Fabrication (or As-Built) Equipment & Package mechanical drawings

- Issued For Construction Instrument Hook-up and Interface drawings

- Approved drawings of in-line instrument items including Control Valves, Pressure Safety Valves, Motor Operated Valves and Emergency Shut Down Valves

- Approved SP item vendor drawings and manual Valve vendor drawings

- Approved Pipe support specification and Piping Material specification

- Issued For Construction Civil and Structural drawings wherever applicable

- In case approved drawings are not available at the time of isometrics issue, the affected items and pipe spools shall be clearly identified as “HOLD” items. In case of any doubt, the “HOLD” issues should be brought to the notice of the Area Engineer / Lead engineer for resolution.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |