| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Pump is located close to equipment from which it taking its suction |  |  |  |
|  | Pump not located under pipe way in process unit. |  |  |  |
|  | Pump motor is toward Rack |  |  |  |
|  | Centre line of Discharge Nozzles on common centerline for rows of pumps as far as possible |  |  |  |
|  | Pump Discharge nozzle minimum 1 m from rack |  |  |  |
|  | Sufficient clear space provided above pump for motor removal |  |  |  |
|  | Sufficient gap between pump foundation and other structures for easy removal of pump. |  |  |  |
|  | Stress Analysis of Pump piping with Supporting is done. |  |  |  |
|  | Pump nozzle loading due to piping is less than vendor allowable |  |  |  |
|  | Strainer in Suction is easily removable. |  |  |  |
|  | Check valve installed in discharge line of centrifugal pump. |  |  |  |
|  | Strainer is located before any reduction in pipe size. |  |  |  |
|  | Suction piping minimized to reduce pressure drop and vapor pockets. |  |  |  |
|  | Pump NPSH Checking done with process department |  |  |  |
|  | Top flat reducer as close as possible to Suction nozzle for front suction nozzle pump. |  |  |  |
|  | Piping Valves readily operable from grade |  |  |  |
|  | Hand wheels of Valves within perimeter of pumps as far as possible. |  |  |  |
|  | Clear Aisle of 1000 mm available between adjacent pumps |  |  |  |
|  | Monorail provided for large pumps if no access possible for mobile equipment. (>= 75KW) |  |  |  |
|  | Pump not located within funded (dyke) tank areas. |  |  |  |
|  | Pumps not located below equipment handling hydrocarbon liquids. |  |  |  |
|  | Water Sprinkler / Spray system provided for pumps handling class A Fluids. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |