| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | General Checks: |  |  |  |
|  | All documents are of latest revisions including any changes or mark-up as part of Technical Querries / Addendum / Design Change Notes issued by client post FEED Completion |  |  |  |
|  | Process Flow Diagram Checks: |  |  |  |
|  | Consideration of all applicable Design cases including SOR, EOR and Alternate case |  |  |  |
|  | Capacity consistent with H&MB |  |  |  |
|  | All major Equipment included |  |  |  |
|  | Stream number consistent with H&MB |  |  |  |
|  | All Properties provided in H&MB |  |  |  |
|  | Any missing PFD / UFD |  |  |  |
|  | Block Flow Diagram Checks: |  |  |  |
|  | Consistent with PFD and Schematic (if developed) |  |  |  |
|  | Process Design Basis Checks: |  |  |  |
|  | All feed cases included |  |  |  |
|  | Site conditions specified as per Tender |  |  |  |
|  | Turndown Specified |  |  |  |
|  | Normal and Design Capacity Specified |  |  |  |
|  | Feed and Product specification |  |  |  |
|  | Applicable Design Pressure / Temperature definition provided |  |  |  |
|  | Demolition scope defined |  |  |  |
|  | Plant Description defined |  |  |  |
|  | Reference To PFDs Available |  |  |  |
|  | Sparing Philosophy defined |  |  |  |
|  | Design Margin specified for all equipment and piping |  |  |  |
|  | Control Philosophy defined |  |  |  |
|  | Isolation Philosophy defined |  |  |  |
|  | Shutdown and ESD Philosophy defined |  |  |  |
|  | Fluid List Document Checks: |  |  |  |
|  | Check all services included |  |  |  |
|  | Process information are included |  |  |  |
|  | HOLDs are identified if any |  |  |  |
|  | Equipment List Checks: |  |  |  |
|  | Consistency with PFDs |  |  |  |
|  | Equipment Details consistent with PFDs / P&IDs |  |  |  |
|  | New and Demolition Equipment are Identified |  |  |  |
|  | Emergency Power consumers identified |  |  |  |
|  | Process Description Checks: |  |  |  |
|  | Inclusion of all Sections |  |  |  |
|  | Material Selection Report / Diagram (MSR / MSD) Checks: |  |  |  |
|  | Inclusion of all Equipment |  |  |  |
|  | Corrosion Allowance Defined |  |  |  |
|  | NACE, IBR (If applicable), Special Lining or coating requirement identified |  |  |  |
|  | Design Pressure & Temperature (Dp-Dt) Diagram Checks: |  |  |  |
|  | Drawing availability for ISBL and OSBL |  |  |  |
|  | DP-DT defined as per project design pressure-temperature philosophy |  |  |  |
|  | Piping Material Specification Checks: |  |  |  |
|  | Specification Availability for Process and Utility area |  |  |  |
|  | Consistency with Fluid List / MSD |  |  |  |
|  | Codes, Standards And Client Specific Procedures: |  |  |  |
|  | Read all applicable codes, standards and client specific procedures are defined and available |  |  |  |
|  | P&ID Checks: |  |  |  |
|  | HAZOP Recommendations incorporated |  |  |  |
|  | SIL Actions are Closed |  |  |  |
|  | Model Review comments incorporated |  |  |  |
|  | Symbols consistent with Legend Sheet |  |  |  |
|  | Consistent with Line Sizing and Hydraulics Calculations |  |  |  |
|  | Consistency between interconnecting P&IDs |  |  |  |
|  | Critical Utility System Designed Correctly e.g. Steam and Condensate System; Closed Cooling Water System etc. |  |  |  |
|  | Uniform tag numbering followed for Equipment, Instruments, Lines, Piping Sp Item, etc. |  |  |  |
|  | Correct references of NOTES available |  |  |  |
|  | Correct references of HOLDS available |  |  |  |
|  | Correct Control Loop shown |  |  |  |
|  | Trim Numbers for Equipment defined (If not defined, put HOLD – “To be allotted during Detail design”.) |  |  |  |
|  | Proper indication of Battery Limit (BL) and Isolation arrangement (e.g. Block and Bleed) followed |  |  |  |
|  | Piping Material Specification defined as per MSD |  |  |  |
|  | Insulation Type and Thickness mentioned |  |  |  |
|  | Slope, Free Draining requirements are defined |  |  |  |
|  | Demolition marked as per Scope |  |  |  |
|  | Chemical Dosing and Make-up Systems identified and shown |  |  |  |
|  | Size of vents and drains are defined (if not defined, put HOLD – “To be finalized during Detail design”.) |  |  |  |
|  | Adequate Safety Shower / Eye wash considered |  |  |  |
|  | Adequate Utility Stations considered |  |  |  |
|  | Value Engineering Comments incorporated |  |  |  |
|  | EIA Comments incorporated |  |  |  |
|  | Adequacy check report of existing utility system |  |  |  |
|  | Missing P&IDs if any |  |  |  |
|  | Vendor package scope and Battery Limit (Piping and Instrumentation) defined properly |  |  |  |
|  | Equipment Datasheet Checks: |  |  |  |
|  | Process data consistent with PFD |  |  |  |
|  | Nozzle sized consistent with P&ID |  |  |  |
|  | Consistent with Sizing Calculations |  |  |  |
|  | Material of Construction consistent with MSD |  |  |  |
|  | Design Pressure / Temperature consistent with DPT Diagram |  |  |  |
|  | Corrosion Allowance mentioned as per MSD |  |  |  |
|  | Package Datasheet Checks: |  |  |  |
|  | Consistent with Vendor Scope of Design and Supply |  |  |  |
|  | Process Guarantee included |  |  |  |
|  | Material of Construction as per MSD |  |  |  |
|  | Battery Limit marked clearly as per definition |  |  |  |
|  | Applicable relevant codes, standards and client specific procedures followed |  |  |  |
|  | Control Narrative Checks: |  |  |  |
|  | Consistent with Design Basis |  |  |  |
|  | Consistent with P&IDs |  |  |  |
|  | Hazop & SIL recommendation incorporated |  |  |  |
|  | Cause & Effect Diagram Checks: |  |  |  |
|  | Consistent with P&ID |  |  |  |
|  | Consistent with Emergency Shutdown (ESD) Philosophy |  |  |  |
|  | Separate system for Process Shutdown (PSD) and Emergency Shutdown (ESD) |  |  |  |
|  | ESD levels configured as per Philosophy and project standards |  |  |  |
|  | Additional interlock added as per Hazop and SIL |  |  |  |
|  | Instrument Process Data Sheet Checks: |  |  |  |
|  | All IPDS included as per P&ID (or Instrument Index) |  |  |  |
|  | Process data consistent with PFD |  |  |  |
|  | Material of Construction consistent with MSD |  |  |  |
|  | Design Pressure-Temperature consistent with DPT Diagram |  |  |  |
|  | Correct flow meter type selected for operating fluid and consistent with P&ID |  |  |  |
|  | Additional instruments added as per Hazop and SIL |  |  |  |
|  | Normal / Rated / Minimum capacity defined as per Process Description and P&IDs |  |  |  |
|  | General Checks: |  |  |  |
|  | Utility network sized correctly e.g. Steam and Condensate; Cooling Water etc. |  |  |  |
|  | P&IDs are properly designed as per client specifications |  |  |  |
|  | Utility Summary prepared and followed consistently |  |  |  |
|  | Existing Utility header adequacy check |  |  |  |
|  | Utility Summary Checks: |  |  |  |
|  | All Utilities are included (including for Steam and Condensate for Tracing) |  |  |  |
|  | Instrument Air consumption values are checked for all Pneumatic operated Valves |  |  |  |
|  | Balance between incoming and outgoing utility lines are consistent (i.e. Cooling Water etc.) |  |  |  |
|  | Tie-in Schedule Checks: |  |  |  |
|  | Consistent with Tie-in P&IDs |  |  |  |
|  | Consistent with Demolition P&IDs |  |  |  |
|  | Consistent with Scope of Work |  |  |  |
|  | Line Designation Table (LDT) Checks: |  |  |  |
|  | Consistent with P&IDs |  |  |  |
|  | Consistent with DPT Diagram |  |  |  |
|  | Consistent with PMS |  |  |  |
|  | Type of pressure test (Hydro / Pneumatic) defined correctly |  |  |  |
|  | Relief Load Summary Checks: |  |  |  |
|  | Consistent with PSVs shown on P&IDs |  |  |  |
|  | Applicable Relief Scenarios Identified |  |  |  |
|  | Relief load calculated for all applicable cases and selected for governing case |  |  |  |
|  | Alarm List Checks: |  |  |  |
|  | Consistent with P&IDs |  |  |  |
|  | Set point defined correctly |  |  |  |
|  | Piping Specialty Item List Checks: |  |  |  |
|  | Consistent with P&IDs |  |  |  |
|  | Process data consistent with Hydraulics |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |