| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| - IFR, IFH and IFC Revision: |  |  |  |  |
| --- | --- | --- | --- | --- |
| S. No. | Check Items | Check |  | Remarks |
|  |  | YES | NO |  |
|  | Document Management / Misc. Technical: |  |  |  |
|  | Drawing Title, Number & Revision |  |  |  |
|  | Equipment Title Blocks, Design/Oper. Conditions, Vessel Trims |  |  |  |
|  | Equipment Names & Numbers |  |  |  |
|  | Specific Notes |  |  |  |
|  | IDC Comments on Previous Issue |  |  |  |
|  | Client Comments on previous Issue |  |  |  |
|  | Operability: |  |  |  |
|  | Normal Operation |  |  |  |
|  | Turndown Operation |  |  |  |
|  | Control Loops |  |  |  |
|  | PCS / PSD Interfaces |  |  |  |
|  | Manual / Auto & Local / Control Room Operation |  |  |  |
|  | Motor Control & Indication Requirements |  |  |  |
|  | Isolation for Operability |  |  |  |
|  | Safety Checks: |  |  |  |
|  | Relief Valve Location / Set Pressure / Size & Sizing Case |  |  |  |
|  | Relief Valve Inlet / Outlet Line Sizes & Valve Interlocks |  |  |  |
|  | ESDV Location / Failure Mode / Test Facilities for Riser Valves |  |  |  |
|  | Blowdown Facilities: Location / Inlet/Outlet Line Sizes |  |  |  |
|  | Low Temperature Considerations / Thermal Sleeves |  |  |  |
|  | Process Trips: Location / Requirement |  |  |  |
|  | LO / LC Requirements for isolation Valves |  |  |  |
|  | HAZOP Comments |  |  |  |
|  | Hydrate Inhibition Requirements |  |  |  |
|  | Insulation Requirements (PP, Winterization, Acoustic) |  |  |  |
|  | Layout Requirements (Elevations, Line Slopes etc.) |  |  |  |
|  | Liquid Containment / Bunding Requirements |  |  |  |
|  | Syphoning / Vacuum Break Requirements |  |  |  |
|  | Filters & Strainers |  |  |  |
|  | Certification Comments |  |  |  |
|  | Start-Up / Shutdown & Maintenance Checks: |  |  |  |
|  | Isolation Requirements: DBB / Spec Blinds / Interlocks |  |  |  |
|  | Depressurization: Manual Facilities, Flare / Atmos. Vent |  |  |  |
|  | Purging Requirements: Connection / Pressure Purging |  |  |  |
|  | Re-pressurization: Manual / Auto Facilities |  |  |  |
|  | Start-up Bypasses & Warm-up Facilities |  |  |  |
|  | Drainage Facilities |  |  |  |
|  | Solids Removal |  |  |  |
|  | Static Equipment Checks: |  |  |  |
|  | All required connections (i.e nozzles) on equipment are shown? |  |  |  |
|  | Requirement of PSV / breather valve / Rupture disc as applicable is shown? |  |  |  |
|  | Requirement of agitator is shown? |  |  |  |
|  | Requirement of coils / jackets is shown? |  |  |  |
|  | All instrumentation on equipment shown? |  |  |  |
|  | If minimum elevations required from ground to bottom tan or bottom of horizontal vessel is shown, if required? |  |  |  |
|  | For S&T heat exchangers, the shell and tube streams matching with PFD? |  |  |  |
|  | Pumps Checks: |  |  |  |
|  | Type of pump properly represented? |  |  |  |
|  | Type of driving equipment is shown? (electric motor, air operated, steam turbine) |  |  |  |
|  | Requirement of VFD identified, if needed? |  |  |  |
|  | Seal flush arrangement, API cooling plan, casing vent and drain identified? |  |  |  |
|  | Are minimum flow by-pass requirements identified (restriction orifice or flow control valve?) |  |  |  |
|  | For metering pumps, are pulsation dampeners shown? |  |  |  |
|  | Compressors Checks: |  |  |  |
|  | Type of compressor shown correctly? |  |  |  |
|  | Special suction piping arrangement, if any, shown? |  |  |  |
|  | Type of driving equipment shown? |  |  |  |
|  | Check valve requirement marked? |  |  |  |
|  | Knock out drums provided upstream of process compressors? |  |  |  |
|  | Utility connections shown to compressors, as required? |  |  |  |
|  | General Piping Checks: |  |  |  |
|  | Process & Utility Flowlines with Directional Arrows |  |  |  |
|  | Line Sizes / Reducers at Line Size Changes |  |  |  |
|  | Piping Specifications / Valve Types / Flange Ratings |  |  |  |
|  | Has piping specs been verified with Material Selection Diagram (MSD)? |  |  |  |
|  | Has the flow been cross checked with PFD? |  |  |  |
|  | Piping Special Items & Numbers |  |  |  |
|  | Insulation (Heat Conservation etc.) |  |  |  |
|  | Piping Spec. / Design Code & Module Breaks / Line Numbers |  |  |  |
|  | Details of Headers & Distribution |  |  |  |
|  | Piping Supports for Vibration / Slugging / Noise |  |  |  |
|  | Hook-up Requirments |  |  |  |
|  | Line Pickling Requirements |  |  |  |
|  | Two Phase flow identified? |  |  |  |
|  | Line slopes / no pockets identified? |  |  |  |
|  | Instrumentation Checks: |  |  |  |
|  | Control Valve Sizes / Handwheels / Bypass etc. |  |  |  |
|  | Control Valves / On-off valves failure position shown? |  |  |  |
|  | Level Bridles |  |  |  |
|  | Alarm & Trip Set Points |  |  |  |
|  | Instrument Tag Numbers |  |  |  |
|  | Type of instruments selected properly (e.g. flow instruments)? |  |  |  |
|  | Flow meters upstream / downstream requirements shown? |  |  |  |
|  | All instrumentation tapping arrangements shown as per instrumentation Line Tap assembly (LTA)? |  |  |  |
|  | Requirement of handwheel and valve open / close position interlock / indication in control room are marked? |  |  |  |
|  | For air cooled heat exchangers, control of motors (VFD, auto variable pitch) or bypass control, etc. shown? |  |  |  |
|  | Metallurgy: |  |  |  |
|  | Corrosion / Erosion |  |  |  |
|  | Corrosion Inhibition |  |  |  |
|  | Chloride Stress Cracking (304 SS / 316 SS) |  |  |  |
|  | Low Temperatures |  |  |  |
|  | Chemicals / Analytical: |  |  |  |
|  | Injection Points for: Methanol, CI, Antifoam etc. |  |  |  |
|  | Sample Points |  |  |  |
|  | Monitoring (Coupons, Probes) |  |  |  |
|  | Consistency Checks: |  |  |  |
|  | Process Data Sheets (Nozzle Schedule Etc.) |  |  |  |
|  | Piping Specifications |  |  |  |
|  | Philosophies |  |  |  |
|  | Clients Specifications |  |  |  |
|  | Holds List |  |  |  |
|  | Symbology (Legend Sheet Accuracy) |  |  |  |
|  | Termination Point (TP's) for Vendor Drawings |  |  |  |
|  | Checked Process Calculations (Line Sizing / CV's / PSV’s) |  |  |  |
|  | Equipment Details (Vessel internal etc.) |  |  |  |
|  | Equipment drains / vents shown. |  |  |  |
|  | Vendor Information incorporation |  |  |  |
|  | Packages Items representation as box? |  |  |  |

| - IFC Issue: |  |  |  |  |
| --- | --- | --- | --- | --- |
| S. No. | Check Items | Check |  | Remarks |
|  |  | YES | NO |  |
|  | Document Management / Misc. Technical: |  |  |  |
|  | Drawing Title, Number & Revision |  |  |  |
|  | All Holds removed? (no holds in IFC Issue should be present) |  |  |  |
|  | Control valves sizes incorporated? |  |  |  |
|  | Pump suction / discharge sizes incorporated? |  |  |  |
|  | Vendor information incorporated for all ordered items? For packages represented properly: |  |  |  |
|  | - tie-ins matching with vendor P&ID? |  |  |  |
|  | - Vendor instrumentation repeat signals shown properly to client DCS? |  |  |  |
|  | - lines sizes / isolation valves at vendor battery limit shown properly? |  |  |  |
|  | 3D Model review (if applicable) comments incorporated? |  |  |  |
|  | For distribution P&ID's- line tap offs sequence properly shown as per piping layout? |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |