| Project name : |  |  |
| --- | --- | --- |
| Project No.: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Project Number |  |  |  |
|  | Project Name |  |  |  |
|  | TEMA Type |  |  |  |
|  | Heat Duty Q,Kcal/hr |  |  |  |
|  | Surface Area A, m2 |  |  |  |
|  | Design Margin on Area % |  |  |  |
|  | Overall, H.T. Coeff. UD Kcal/hr, m2, 0C |  |  |  |
|  | Mean Temp, Diff 0C |  |  |  |
|  | Fluid Shell/Tube |  |  |  |
|  | Allowable Pressure Drop Shell/Tube kg/cm2 |  |  |  |
|  | Calculated tube side and shell side pressure drop kg/cm2 |  |  |  |
|  | MOC Shell/Tube |  |  |  |
|  | Fouling Factor, Shell hr m2 C / kcal |  |  |  |
|  | Fouling Factor, Tube hr m2 C / kcal |  |  |  |
|  | Ensure that standard pipe sizes have been used for shell ID (before lining) less or equal to 16", Check with Mechanical for ID to be taken |  |  |  |
|  | Check that all process conditions are taken care in the output of the designed heat exchanger |  |  |  |
|  | Ensure the following: |  |  |  |
|  | Nozzles on opposite sides is compatible with even number of baffles |  |  |  |
|  | Nozzle on same side is compatible with odd number of baffles |  |  |  |
|  | Baffle spacing is in the range of 20% to 100% of the shell diameter |  |  |  |
|  | Ensure end baffle spacing is adjusted to suit selected nozzle size |  |  |  |
|  | Ensure baffle cut orientation is properly selected (see HTRI help menu) |  |  |  |
|  | Ensure that standard tube lengths are taken. |  |  |  |
|  | Is the nozzle location in HX as per the P&ID |  |  |  |
|  | For segmental baffles Are window flow & cross flow vel nearly same |  |  |  |
|  | For NTIW baffles Is ratio of window vel to cross flow vel between 2 & 3 |  |  |  |
|  | Cooling water velocity in m/sec, if applicable (CW velocity should be more or equal to 1m/sec) |  |  |  |
|  | Has shell side pressure drop been optimized |  |  |  |
|  | Has tube side pressure drop been optimized |  |  |  |
|  | Is ratio of nozzle pressure drop to total pressure drop not too high for: (Generally restrict nozzle side pressure drop for both shell and tube sides to 20% of the total) |  |  |  |
|  | Tube side |  |  |  |
|  | Shell side |  |  |  |
|  | Whether flow velocities on shell and tube side fall in the valid range |  |  |  |
|  | Has the requirement of impingement (shell side) plate/vapor belt been checked with respect to nozzle pv2? (provide impingement plate for condensing vapor) |  |  |  |
|  | Liquid Flow - Limit nozzle pv2 to about 2000 lb/ft sec2 (3000 kg/m, s2) |  |  |  |
|  | Boiling liquids - 744 kg/m s2 |  |  |  |
|  | Gas Flow - Check that nozzle vel is less than 20% of acoustic velocity |  |  |  |
|  | Has HX been checked for flow induced vibration |  |  |  |
|  | For condensers HTRI uses Resistance Proportion Method by default. For selecting other options, please use HELP menu |  |  |  |
|  | Condenser Area ratio shall be in excess of 10% over the required area in all the cases except instructed otherwise |  |  |  |
|  | Check out the message displayed in the output of the exchanger design. This will help to improve the design of condenser |  |  |  |
|  | Check that heat curve data is properly entered and from reliable sources for vaporizers/condensers |  |  |  |

Allowable Fractions & Radial Clearances:

| * | Allowable flow fractions of various streams on the shell side are as follows: |  |  |
| --- | --- | --- | --- |
|  | TYPE OF STREAM | FRACTION OF TOTAL SHELL SIDE FLOW |  |
|  | B-Stream | Main Cross flow stream with bundle | At least 0.6 (for turbulent flow) At least 0.4 (For laminar flow) |
|  | C-Stream | Bundle to shell cross flow bypass stream | Normally less than 0.1 |
|  | A-Stream | Tube to bundle hole leakage stream | Usually smaller for multisegmented baffle |
|  | E-Stream | Baffle to shell leakage stream | Should be less than 0.15 |
|  | F-Stream | Tube pass partition bypass stream | Should not exceed 0.10 |
|  | Note: Applicable only where no phase change occurs |  |  |
| ** | Use the following Bundle/Shell ID (after lining where applicable) radial clearances |  |  |
|  | HEAD TYPE | RADIAL CLEARANCES |  |
|  | M type, Fixed tube sheet | 10 mm |  |
|  | U type, Rear Head | 10 mm |  |
|  | S type, Rear Head | 30 mm |  |
|  | P Type, Rear Head | 22 mm |  |
|  | T Type, Rear Head | Contact MQ |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- This checklist shall be filled and produced along with the HTRI output

- For placing of fluids on shell side or tube side see relevant chapter in Handbook

- Fouling factors for shell/tube side fluids to be as per project specification; Alternatively, refer handbook chapter.

- For tubes pattern and pitch refer handbook chapter.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |