| Project Name : |  |  |
| --- | --- | --- |
| Equipment Name: |  | Tag No.: |
| Document No.: | REV. : | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | GENERAL |  |  |  |
|  | Ensure that latest revisions of the following documents are available before initiating review. - Mechanical Datasheet - Approved Design Calculations - P&ID - PMS - Project Specifications/Standards - Standard Drawings - Plot Plan - Civil Foundation Drawing - Nozzle orientation inputs from piping - Pipe support cleat inputs from piping - CP cleat inputs from metallurgy (if required) |  |  |  |
|  | GA DRAWING |  |  |  |
|  | Job/Req/Item Nos./Client/Consultant names indicated. |  |  |  |
|  | All major components mounted on skid are shown. |  |  |  |
|  | Check correspondence to Datasheet for: |  |  |  |
|  | a) Pump description, model, nozzle size, rating & facing |  |  |  |
|  | b) Direction of rotation, Nozzle orientation Size, rating, facing for casing drain, casing vent, cooling, flushing, heating connections. |  |  |  |
|  | c) Motor rating, rated speed, frame size, Make |  |  |  |
|  | d) Coupling model |  |  |  |
|  | Auxiliary piping details |  |  |  |
|  | Check that the following dimensions are shown: |  |  |  |
|  | a) Elevation of pump centerline from bottom of base plate. Motor centerline shall be matched with pump centerline. |  |  |  |
|  | b) Location of centerline of discharge with respect to edge of base plate. |  |  |  |
|  | c) Location of pocket for foundation bolt with respect to edge of base plate. |  |  |  |
|  | d) Overall size of base plate, overall size of unit |  |  |  |
|  | e) Location (all three co-ordinates) for main and auxiliary flanges at the point of termination. |  |  |  |
|  | f) Location & size of motor mounting holes on the base plate & motor shaft end dimensions and shall be verified as per motor drawing. |  |  |  |
|  | g) Distance between shaft ends or spacer length of coupling. |  |  |  |
|  | h) Pump shaft & Motor shaft end details. |  |  |  |
|  | i) Location of seal flushing system (if applicable) |  |  |  |
|  | j) Elevation difference between separate skids (if applicable) |  |  |  |
|  | Other dimensions for sump pumps |  |  |  |
|  | a) Bottom of Mounting plate to Impeller C. L. & sump bottom. |  |  |  |
|  | b) Clearance between suction strainer bottom and sump bottom. |  |  |  |
|  | c) Min/Max liquid level w.r.t. Impeller C.L/sump bottom |  |  |  |
|  | d) Size of opening in sump cover. |  |  |  |
|  | For vertical barrel pump |  |  |  |
|  | a) Outside diameter of barrel |  |  |  |
|  | b) Pit dimension |  |  |  |
|  | c) Insulation thickness, if insulation is required for low temperature service |  |  |  |
|  | d) Vertical clear height for pump removal |  |  |  |
|  | Static weight of individual equipment, Total static load, Dynamic load for critical, high-speed machines. Heaviest component weight for maintenance shall be indicated. |  |  |  |
|  | Size of foundation bolt hole and number of holes on base plate. |  |  |  |
|  | Size of foundation bolt, size of pocket for foundation bolt, Pitch of pockets, Projection of foundation bolt above base plate. |  |  |  |
|  | Grout thickness & Top of grout elevation level. |  |  |  |
|  | Grout hole and vent hole, when required by vendor. |  |  |  |
|  | Coupling guard is shown. |  |  |  |
|  | Provision of jack screw, alignment positioning screw for heavy components. |  |  |  |
|  | Drain pan in base plate, size of drain hole. |  |  |  |
|  | Type of companion flange is shown when supplied by vendor. |  |  |  |
|  | Motor terminal box orientation & motor lifting arrangement. |  |  |  |
|  | Allowable Forces & Moments on pump nozzles. |  |  |  |
|  | Revision mark shown near all new revisions. |  |  |  |
|  | Check whether the center of gravity is marked with dimension. |  |  |  |
|  | Check whether all tie in points including instrument junction box is shown. |  |  |  |
|  | Baseplate material shall be indicated |  |  |  |
|  | Check North direction is marked. |  |  |  |
|  | Check Earthing boss & Lifting lugs location / details |  |  |  |
|  | Check drawing title block as per project standard format. |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |