| Project name : |  |
| --- | --- |
| Package Name: | Tag No.: |
| Vendor: | Date : |

| S. No. | Check Items | Check |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | YES | NO |  |
|  | Serial no. in name plate |  |  |  |
|  | Model no. in name plate |  |  |  |
|  | Rated duty conditions in name plate |  |  |  |
|  | Name plate material and construction |  |  |  |
|  | Check tags, model no., tag plates, duty of individual components w.r.t data sheet |  |  |  |
|  | Check basic constructional details of individual components w.r.t drgs |  |  |  |
|  | Motor and heater ratings on its name plate to match load list provided to us |  |  |  |
|  | Name plate details + OEM's certificate of compliance |  |  |  |
|  | Check overall arrangement w.r.t P&ID and GAD |  |  |  |
|  | Check orientation of arrangement as per GAD |  |  |  |
|  | For any mirror arrangement make sure the tags are as per drgs for individual items |  |  |  |
|  | Check location, size and type of all valves as per drgs |  |  |  |
|  | Check size of all piping and material as per drgs |  |  |  |
|  | Check if individual components are supported rigidly |  |  |  |
|  | Overall dimensions |  |  |  |
|  | Anchor bolt location |  |  |  |
|  | Anchor bolt dia. |  |  |  |
|  | Anchor bolt straddling (for circular frames) |  |  |  |
|  | Base frame stiffeners |  |  |  |
|  | Intermediate frames for rigidity |  |  |  |
|  | Any jacking bolts provided – has this been covered in drg. – does it require insert plate |  |  |  |
|  | If anchor bolts are in vendor's scope - check w.r.t drgs and specs |  |  |  |
|  | Lifting lugs and NDE certificates |  |  |  |
|  | Base frame drain point should be with valve and located away from frame edge to prevent fouling with foundation |  |  |  |
|  | Drain pans to be provided from draining component to drain valve |  |  |  |
|  | Walkway on base frame to be provided with antiskid decking |  |  |  |
|  | Access to all inspection and maintenance points |  |  |  |
|  | Access and space for removal of internal components (identify components) |  |  |  |
|  | All piping TP (Terminal Points) std ANSI flange as per drg |  |  |  |
|  | All TPs to have RF flanges |  |  |  |
|  | All TPs to have flange bolts off-centered or on-centered as per spec |  |  |  |
|  | All TPs to match drg on co-ordinates and orientation |  |  |  |
|  | All TPs to be arranged to have space behind flange for bolting |  |  |  |
|  | All TPs to be skid edge |  |  |  |
|  | All TPs at higher elevation in the middle of skid shall have its piping routed to a side of skid with adequate support |  |  |  |
|  | All TPs to have a support as close as possible to TP |  |  |  |
|  | Location of PSV outlets, local vents to be directed away from operator |  |  |  |
|  | If insulation is applicable and not yet done, check for space for other adjacent component (reduced after the insulation has been done) |  |  |  |
|  | Vertical bolting on clamps etc. to have bolt head on top |  |  |  |
|  | All bolting and clamping in SS |  |  |  |
|  | Check thoroughly for pipe supports – note that the skid will be transported by air / ship / desert environment. Supports to be really sound and rigid. |  |  |  |
|  | Check all cable terminal boxes and JBs |  |  |  |
|  | Check TB / JB numbers from drg |  |  |  |
|  | Check cable terminal numbers from drg |  |  |  |
|  | Check internal cable make, construction with specifications (inscribed on it / ask for certs) |  |  |  |
|  | Check cable numbers and numbering strip construction as per specifications |  |  |  |
|  | Check cable color as per specifications |  |  |  |
|  | Check fire retardant / resistant property |  |  |  |
|  | Check cable trenching, armoring, clamping and supporting as per specifications |  |  |  |
|  | Check cable glands – material, ATEX, dual certs as per specifications |  |  |  |
|  | Check cable gland size (for interfaces) as per drgs |  |  |  |
|  | Cables to be routed in safe manner, check for loose cables, trip hazards, mech guards etc. |  |  |  |
|  | Check cable tray construction as per specifications |  |  |  |
|  | Check space available for bringing in our cables, how the routing has been identified w.r.t location and number on drg. |  |  |  |
|  | Check cable entry – as per specifications /drgs |  |  |  |
|  | Check removable gland plates as per spec – no. of cables, size, location and space for cable entry |  |  |  |
|  | Check scope of supply for glands |  |  |  |
|  | Check tag plate construction for all instruments |  |  |  |
|  | Check ferruling as per specifications |  |  |  |
|  | Heat tracing cable and its JB |  |  |  |
|  | Material of gland plates |  |  |  |
|  | Material of TBs and JBs |  |  |  |
|  | ATEX certificate for cables, TBs and JBs |  |  |  |
|  | IP rating for TBs and JBs |  |  |  |
|  | Ex proof rating for TBs and JBs |  |  |  |
|  | Check provision of earthing bosses on skid |  |  |  |
|  | Check earthing of internal components and how they are grouped and terminated |  |  |  |
|  | Check earthing lugs, strips, wiring and jumpers as needed |  |  |  |
|  | Check all instrument tags |  |  |  |
|  | Check type of instruments as per data sheets |  |  |  |
|  | Glycol / other liquid fills for gauges |  |  |  |
|  | Check range of all instruments |  |  |  |
|  | Ask for calibration certificates |  |  |  |
|  | Check mounting details and hook up as per specifications |  |  |  |
|  | Check for SBB and DBB provisions as per specifications |  |  |  |
|  | Check for 3/5 manifolds as per specifications |  |  |  |
|  | Check for root isolation valves |  |  |  |
|  | Check PG dial gauge size as per specifications |  |  |  |
|  | Transmitters to have indicators |  |  |  |
|  | Cable gland size to match drgs |  |  |  |
|  | IP rating of instruments |  |  |  |
|  | Ex proof rating of instruments |  |  |  |
|  | ATEX certs for instruments |  |  |  |
|  | All instrument displays / gauges are visible from grade |  |  |  |
|  | All instruments to be easily accessible for inspection / maintenance |  |  |  |
|  | Check enclosure construction as per drg |  |  |  |
|  | Enclosure IP rating |  |  |  |
|  | Enclosure maintenance and inspection points, access to internals |  |  |  |
|  | Removal of enclosure |  |  |  |
|  | Enclosure heat dissipation from within / ventilation requirements |  |  |  |
|  | Enclosure acoustic and/or thermal insulation specification to be verified as per specifications |  |  |  |
|  | Any internal lighting requirements, provision for temporary lighting points for inspection? |  |  |  |
|  | Vent or exhaust from enclosure to be routed to safe point (if local) or piped to TP at skid edge |  |  |  |
|  | Check scope coverage |  |  |  |
|  | Check what components are loose supplied |  |  |  |
|  | Check how loose components are to be installed and connected with main package at site (example connection flange spec, cable connection, matching joints etc.) |  |  |  |
|  | Check how these loose components are identified (tag no., part no., identification of these in drgs and packing list etc.) |  |  |  |
|  | Check intra-skid piping and valving if in vendor's scope - material, construction, flange connections as per spec. How these are supplied loose, tagged and identified in drawings and on material? |  |  |  |
|  | How are the heat tracing cable and insulation supplied? |  |  |  |
|  | Insulation for any other components - thermal or PP - how are they supplied? |  |  |  |
|  | If insulation is by PUL, the cleats should have been welded on the component |  |  |  |
|  | Check start-up spares list |  |  |  |
|  | Check lifting beam / spreader bar construction |  |  |  |
|  | Check test certificate of lifting beam / spreader bar |  |  |  |
|  | If possible, ask vendor to demonstrate use of lift beam / spreader bar |  |  |  |
|  | Check special tools and its lockable container (manuals and drgs to be submitted together with) |  |  |  |
|  | Check if all pressure components have cleared hydro-test and other NDE; verify few certs |  |  |  |
|  | Check if any heat treatment is applicable and verify the cert |  |  |  |
|  | Check PMI certificates |  |  |  |
|  | Check compliance certificates of some key components |  |  |  |

Notes:

- This checklist is no way comprehensive. Based on specific project conditions/equipment some points as deemed necessary can be added / deleted. Addition / deletion of points, if any, needs to be taken up in consultation with Lead Engineer / Technical Specialist / Discipline Manager.

- If     is not filled against any item, please indicate N/A or else provide remarks.

- After release of the respective documents, the completed check list shall be retained by Discipline Engineer for future reference and for technical / quality audit.

- Check points shall be filled by job engineer and checked/approved by Lead/Tech. Specialist.

- The final approved Checklist shall be attached with the respective document if required.

| REV. | SIGN | DATE | SIGN | DATE | REMARKS |
| --- | --- | --- | --- | --- | --- |
|  | JOB ENGINEER |  | LEAD ENGINEER |  |  |